## (Lab Name) LABORATORY

# EXPOSURE CONTROL PLAN

- Introduction…………………………………………………..2

- Roles & Responsibilities……………………………………3

- Exposure Determination……………………………………3

- Methods of Implementation & Control…………………….3

- Hepatitis B Vaccination……………………………………..7

- Post Exposure Evaluation & Follow-up……………………8

- Incident Reporting…………………………………………...9

- Training……………………………………………………….9

- Recordkeeping……………………………………………….10

Appendix A.  Glossary of Terms……………………………….13

Appendix B.  Spill Cleanup Procedures..……………………..16

Appendix C.  HBV Declination Form…………………………..17

Appendix D.  Sharps Injury Log………………………………..18

- INTRODUCTION

  - Purpose

This plan is designed to eliminate employee exposure to bloodborne pathogens. All human blood and other potentially infectious materials (OPIM) are considered to be infectious for Human Immunodeficiency Virus (HIV), Hepatitis B virus (HBV), and other bloodborne pathogens, and will be treated as if infectious, i.e. with universal precautions. Since animal blood is not readily distinguished from human blood by appearance, this document's guidance for handling and disposal of human blood and sharps is recommended for all blood.  Definitions relevant to this document are found in Appendix A.

  - Scope

This exposure plan is mandatory for all University of Utah laboratory employees who have occupational exposure to human blood and OPIM.  This template has been developed by the Department of Occupational and Environmental Health and Safety (OEHS) and the plan must be customized so that it provides lab-specific provisions to identify and protect all personnel who may be at risk of exposure.  This plan must be updated at least annually, and whenever there are changes in laboratory procedures that may change a worker’s exposure.  A copy of this plan must remain in the lab, and must be accessible to lab personnel.

  - Policy

The (Name of Laboratory) laboratory is committed to providing a safe and healthful work environment for our entire staff.  In pursuit of this endeavor, the following Exposure Control Plan (ECP) is provided in accordance with OSHA standard 29 CFR 1910.1030, “Occupational Exposure to Bloodborne Pathogens.”

The ECP is a key document to assist our lab in implementing and ensuring compliance with the standard, thereby protecting our employees. This ECP includes:

- Employee exposure determination

- Methods of control to prevent worker exposure

- Provisions for hepatitis B vaccination

- Post-exposure evaluation & follow-up

- Employee training and hazard communication

- Recordkeeping

Implementation of these elements of the ECP is discussed in the subsequent pages of this document.

- ROLES AND RESPONSIBILITIES

2.1	(Name of Individual) is responsible for the implementation of this ECP.

2.2	(Name of Individual) will maintain, review, and update this ECP at least annually, and whenever necessary to include new or modified tasks and procedures.

2.3	Those employees who have occupational exposure to blood or OPIM must comply with the procedures and work practices outlined in this ECP.

2.4	(Name of Individual) will maintain and provide all necessary personal protective equipment (PPE), engineering controls (e.g., sharps containers), labels, and red bags as required by the standard.  This will include ensuring that adequate supplies of the aforementioned equipment are available in the appropriate sizes.

2.5	(Name of Individual) will be responsible for ensuring that all medical actions required are performed and that appropriate employee health and OSHA records are maintained.

2.6	(Name of Individual) will be responsible for training, documentation of training, and making the written ECP available to employees, OSHA, and NIOSH representatives.

#### 3.	EMPLOYEE EXPOSURE DETERMINATION

3.1	Following is a list of tasks or procedures where exposure to blood or OPIM may occur, and the names of those individuals who have real or potential exposure as a result of performing those tasks:

| Task or Procedure | Employees with Real or Potential Exposure | Job Title(s) |
| --- | --- | --- |
| Task #1 (Example) Phlebotomy, Vena Puncture | (Employee #1 Name) (Employee #2 Name) | Phlebotomist Lab Technician |

- METHODS OF IMPLEMENTATION AND CONTROL

4.1  	Universal Precautions

All employees will utilize universal precautions when handling blood and OPIM.  For tasks in which differentiation between body fluid types is difficult or impossible, all body fluids will be considered potentially infectious materials.

4.2	Exposure Control Plan

Employees covered by the bloodborne pathogens standard receive an explanation of this ECP during their initial training session. It will also be reviewed in their annual refresher training.  All employees have an opportunity to review this plan at any time during their work shifts.  A copy of the plan will be located in the laboratory in the following location:  (Location where the ECP will be located in the lab).

  - Engineering Controls, Work Practices, and Personal Protective Equipment (PPE)

    - Engineering Controls.  Laboratory personnel will use engineering controls whenever possible to prevent exposure.  All aerosol-producing tasks (i.e. sonicating or vortex mixing), or tasks that may result in splashing or spraying, will be conducted in a certified biological safety cabinet.  Other engineering controls such as sharps disposal containers, self-sheathing needles, and non-glass capillary tubes will be used as needed depending on the task being performed.

    - Work Practice Controls.  Work practice controls include policies and procedures to minimize exposure, and should be used in conjunction with engineering controls.  Work practice controls include hand washing, safe needle practices (no recapping), no eating or drinking in the lab, and no mouth pipetting.

    - Personal Protective Equipment.  PPE will be used to prevent or minimize exposure to bloodborne pathogens, but should only be used when exposure remains after all reasonable engineering controls and work practice controls are in place.  PPE includes such items as gloves and lab coats.  For procedures where splashing may occur (spill cleanup), eye protection and face protection (masks) must be used to prevent exposure.  Face shields may be used as an alternative to the safety glasses / mask combination.

  - Following are the specific engineering controls and PPE that will be used by employees to minimize exposure to blood and OPIM for each task outlined in paragraph 3.1 above:

| Task # (See 3.1 Above) | Engineering Control Measures |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bench Top Splash Shield | Biohazard Waste Bags | Biosafety Cabinet | HEPA Vacuum Filters | Plastic Transport Containers | Self-Sheathing Needles | Non-Glass Capillary Tubes | Other (Please Specify) |
| Task #1 (example) |  | X |  |  |  | X | X |  |

| Task # (See 3.1 Above) | Required Personal Protective Equipment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Lab Coat | Gloves | Safety Glasses with Side Shields | Surgical Mask | Solid Front Gown | Sleeve Covers | Booties | Respiratory Protection (Contact OEHS) | Face Shield | Other (Please Specify) |
| Task #1 (example) | X | X |  |  |  |  |  |  |  |  |

- Specific Control Measures

Sharps disposal containers are inspected and maintained or replaced by (Name of Individual) whenever necessary to prevent overfilling.

This laboratory identifies the need for changes in engineering control and work practices through our weekly lab meeting.

If needles or syringes are used, we evaluate new procedures or new products regularly to improve safety by searching various venders and asking for samples to try.

Both front line workers and management officials are involved in this process: Employees are involved through our weekly lab meeting where there is an open forum to bring up any issues or potential issues in the lab.

PPE is located (describe locations of PPE in the lab).

All employees using PPE must observe the following precautions:

Wash hands immediately or as soon as feasible after removal of gloves or other PPE.

Remove PPE after it becomes contaminated, and before leaving the work area.

Used PPE may be disposed of (describe method of disposal).

Wear appropriate gloves when it can be reasonably anticipated that there may be hand contact with blood or OPIM, and when handling or touching contaminated items or surfaces; replace gloves if torn, punctured, contaminated, or if their ability to function as a barrier is compromised.

Never wash or decontaminate disposable gloves for reuse.

Wear appropriate face and eye protection when splashes, sprays, spatters, or droplets of blood or OPIM pose a hazard to the eye, nose, or mouth.   Appropriate face and eye protection consists of mask & goggles or a face shield.

Remove immediately or as soon as feasible any garment contaminated by blood or OPIM, in such a way as to avoid contact with the outer surface.

4.6	Housekeeping

Spills of blood or OPIM must be cleaned up using an EPA approved disinfectant (https://www.epa.gov/pesticide-registration/selected-epa-registered-disinfectants) following the procedure described in Appendix B.

Regulated waste is placed in containers which are closable, constructed to contain all contents and prevent leakage, appropriately labeled or color-coded (see Labels), and closed prior to removal to prevent spillage or protrusion of contents during handling.

Sharps containers will be disposed of by filling out the on-line pick up request at www.oehs.utah.edu.  OEHS personnel will pick up full containers and replace them with empty containers.

Other regulated waste must be autoclaved before being disposed of in the municipal waste stream.  Alternatively, OEHS can dispose of biohazard waste for the lab.  OEHS can be contacted using the on-line pick up request.

Contaminated sharps are discarded immediately or as soon as possible in containers that are closable, puncture-resistant, leakproof on sides and bottoms, and labeled or color-coded appropriately. Sharps disposal containers are available through the OEHS on-line pick up request at www.oehs.utah.edu.

Bins and pails (e.g., wash or emesis basins) are cleaned and decontaminated as soon as feasible after visible contamination.

Broken glassware that may be contaminated is picked up using a mechanical device such as tongs, forceps, or a brush and dustpan.

  - Laundry

The following articles, if contaminated, will require laundering:

The University of Utah Hospital Laundry will be used to clean contaminated clothing and other articles that require laundering.

The following laundering requirements must be met:

- Handle contaminated laundry as little as possible, with minimal agitation.

- Place wet contaminated laundry in leak-proof, labeled or color-coded containers before transport to the University Hospital Laundry.

  - Labels

The universal biohazard symbol will be used to mark regulated waste containers, refrigerators containing blood or OPIM, contaminated equipment, specimen containers, and specimen transport containers.  The universal biohazard symbol must be located at the entrance to all laboratory rooms where blood or OPIM is used.  Red bags can be used as a substitute for bags marked with the universal biohazard symbol for regulated waste only.

(Name of Individual) will ensure warning labels are affixed or red bags are used as required if regulated waste or contaminated equipment is brought into the facility.

  - Hepatitis B Vaccination

Information on hepatitis B vaccinations, including safety, benefits, efficacy, methods of administration, and availability, will be provided to employees during annual Bloodborne Pathogens Training.

The hepatitis B vaccination series is available at no cost after training and within 10 days of initial assignment to employees identified in the exposure determination section of this plan (section 3.1). Vaccination is encouraged unless: 1) documentation exists that the employee has previously received the series, 2) antibody testing reveals that the employee is immune, or 3) medical evaluation shows that vaccination is contraindicated.

However, if an employee chooses to decline vaccination, the employee must sign a declination form (see Appendix C).  Employees who decline may request and obtain the vaccination at a later date at no cost. Documentation of refusal of the vaccination is kept (describe location).

Vaccination will be coordinated and paid for through OEHS, as provided by the Student Health Clinic.

Following the medical evaluation, a copy of the health care professional's written opinion will be obtained and provided to the employee. It will be limited to whether the employee requires the hepatitis vaccine, and whether the vaccine was administered.

- Post-Exposure Evaluation and Follow-Up

Should an exposure incident occur, do the following:

Immediately wash affected areas with soap and water.  If eye exposure occurred, immediately flush eyes with water for 10-15 minutes.

Notify lab supervisor or Principal Investigator of the exposure.

Go directly to the Occupational Medicine Clinic at the Redwood Health Center for medical evaluation and follow-up.  Source blood testing will be determined by the physician.

RedMed Employee Health Clinic

200 Central Campus Dr.

Salt Lake City, UT 84112

Phone: (801) 213-3303

Hours: M-TH: 8:00AM – 5:00PM, Friday: 9:00AM – 3:30PM

Redwood Health Center

Occupational Medicine Clinic

1525 West 2100 South

Salt Lake City, UT 84119

Phone: (801) 213-9777

Hours: M-F 8:00AM – 5:00PM

After Hours

Redwood Urgent Care

1525 West 2100 South

Salt Lake City, UT 84119

M-F 5:00PM – 8:00PM

Sat.-Sun.: 9:00AM – 8:00PM

(801) 213-9700

After 8 PM

Emergency Department at University Hospital

(main floor northeast side of the hospital)

50 N. Medical Drive

Salt Lake City, UT 84132

(801) 581-2292

Maps of Occupational Medicine Clinics

UNIVERSITY OF UTAH REDWOOD HEALTH CENTER OCCUPATIONAL MEDICINE 1525 W. 2100 S. Salt Lake City UT, 84119 REDMED EMPLOYEE HEALTH CLINIC 200 Central Campus Dr. Salt Lake City, UT 84112

- Incident Reporting

An employee with a work-related injury or illness must complete the Employer’s First Report of Injury or Illness Form, Form 122, and submit it to the Benefits Department within 24 hours.  This form can be accessed by clicking on “Forms” at www.hr.utah.edu or by calling Benefits at 801- 581-7447.  The Benefits Department is located at 420 Wakara Way Suite 105 Salt Lake City, UT 84108.  All exposures will be reported to OEHS 801-581-6590.  OEHS will ensure that all OSHA reportable cases are reported to UOSH.

Identify and document the source individual (unless it is established that the source is unknown).

After obtaining consent, collect exposed employee's blood as soon as feasible after exposure incident, and test blood for HBV and HIV serological status.

If the employee does not give consent for HIV serological testing during collection of blood for baseline testing, preserve the baseline blood sample for at least 90 days; if the exposed employee elects to have the baseline sample tested during this waiting period, perform testing as soon as feasible.

  - Training

All employees who have occupational exposure to bloodborne pathogens receive training annually by OEHS.

All employees who have occupational exposure to bloodborne pathogens receive training on the epidemiology, symptoms, and transmission of bloodborne pathogen diseases. In addition, the training program covers, at a minimum, the following elements: a copy and explanation of the standard, an explanation of our ECP and how to obtain a copy, an explanation of methods to recognize tasks and other activities that may

involve exposure to blood and OPIM, including what constitutes an

exposure incident, an explanation of the use and limitations of engineering controls, work practices, and PPE, an explanation of the types, uses, location, removal, handling, decontamination, and disposal of PPE, an explanation of the basis for PPE selection, information on the hepatitis B vaccine, including information on its efficacy, safety, method of administration, the benefits of being vaccinated, and that the vaccine will be offered free of charge, information on the appropriate actions to take and persons to contact in an emergency involving blood or OPIM, an explanation of the procedure to follow if an exposure incident occurs, including the method of reporting the incident and the medical follow-up

that will be made available, information on the post-exposure evaluation and follow-up that the employer is required to provide for the employee following an exposure incident, an explanation of the signs and labels and/or color coding required by the standard and used at this facility, an opportunity for interactive questions and answers with the person conducting the training session.

Training materials for this facility are available through OEHS (bldg. 605).

- Recordkeeping

9.1 	Training Records

Training records are completed for each employee upon completion of training. These documents will be kept for at least three years at OEHS (Bldg. 605).

The training records include: Dates of the training sessions, content of the training sessions, the name of the person(s) conducting the training, and names of all persons attending the training sessions.

Employee training records are provided upon request to the employee or the employee's authorized representative within 15 working days. Such requests should be made to OEHS (Bldg. 605).

9.2      Medical Records

Medical records are maintained for each employee with occupational exposure in accordance with 29 CFR 1910.1020, "Access to Employee Exposure and Medical Records."  Medical records, under most circumstances, will be maintained at the Redwood Health Center, Occupational Medicine Clinic.

9.3 	OSHA Recordkeeping

Human resources will evaluate all incident reports to determine if cases meet OSHA’s Recordkeeping Requirements (29 CFR 1904). All percutaneous injuries from contaminated sharps are also recorded in the Sharps Injury Log (Appendix D).

APPENDIX A

### Glossary of Terms

Blood means human or non-human primate blood, blood components, and blood-based products.   

Bloodborne Pathogens means pathogenic microorganisms that are present in human or non-human primate blood and can cause disease in humans. Examples include, but are not limited to, hepatitis B virus (HBV), and human immunodeficiency virus (HIV). 

Clinical Laboratory means a workplace where diagnostic or other screening procedures are performed on blood or other potentially infectious materials. 

Contaminated means the presence or the reasonably anticipated presence of blood or other potentially infectious materials on an item or surface. 

Contaminated Laundry means laundry that has been soiled with blood or other potentially infectious materials or may contain sharps. 

Contaminated Sharps means any contaminated object that can penetrate the skin including, but not limited to, needles, scalpels, broken glass, and broken capillary tubes. 

Decontamination means the use of physical or chemical means to remove, inactivate, or destroy bloodborne pathogens on a surface or item to the point where they are no longer capable of transmitting infectious particles and the surface or item is rendered safe for handling, use, or disposal. 

Engineering Controls means controls (e.g., sharps disposal containers, self-sheathing needles, safer medical devices, such as sharps with engineered sharps injury protections and needleless systems) that isolate or remove the bloodborne pathogens hazard from the workplace. 

Exposure Incident means a specific eye, mouth, other mucous membrane, non-intact skin, or parenteral contact with blood or other potentially infectious materials that results from the performance of job duties. 

Handwashing Facilities means a facility providing an adequate supply of running potable water, soap and single use towels or hot air drying machines. 

Licensed Healthcare Professional is a person whose legally permitted scope of practice allows him or her to independently perform the activities required for Hepatitis B Vaccination and Post-exposure Evaluation and Follow-up. 

HBV means hepatitis B virus. 

HIV means human immunodeficiency virus. 

Needleless systems means a device that does not use needles for: 

(1) The collection of bodily fluids or withdrawal of body fluids after initial venous or arterial access is established; (2) The administration of medication or fluids; or (3) Any other procedure involving the potential for occupational exposure to bloodborne pathogens due to percutaneous injuries from contaminated sharps. 

Occupational Exposure means reasonably anticipated skin, eye, mucous membrane, or parenteral contact with blood or other potentially infectious materials that may result from the performance of job duties. 

Other Potentially Infectious Materials means (1) The following human body fluids: semen, vaginal secretions, cerebrospinal fluid, synovial fluid, pleural fluid, pericardial fluid, peritoneal fluid, amniotic fluid, saliva in dental procedures, any body fluid that is visibly contaminated with blood, and all body fluids in situations where it is difficult or impossible to differentiate between body fluids; (2) Any unfixed tissue or organ (other than intact skin) from a human (living or dead); (3) HIV-containing cell or tissue cultures, organ cultures, and HIV- or HBV-containing culture medium or other solutions; and blood, organs, or other tissues from experimental animals infected with HIV or HBV; (4) All primary human and non-human primate cell explants from tissues and subsequent in vitro passages of human or primate tissue explant cultures, unless characterized by documented, reasonable laboratory testing to be free of HIV, HBV, HCV, and other bloodborne pathogens.   

Parenteral means piercing mucous membranes or the skin barrier through such events as needlesticks, human/non-human primate bites, cuts, and abrasions. 

Personal Protective Equipment is specialized clothing or equipment worn by an employee for protection against a hazard. General work clothes (e.g., uniforms, pants, shirts or blouses) not intended to function as protection against a hazard are not considered to be personal protective equipment. 

Production Facility means a facility engaged in industrial-scale, large-volume or high concentration production of HIV or HBV. 

Regulated Waste means liquid or semi-liquid blood or other potentially infectious materials; contaminated items that would release blood or other potentially infectious materials in a liquid or semi-liquid state if compressed; items that are caked with dried blood or other potentially infectious materials and are capable of releasing these materials during handling; contaminated sharps; and pathological and microbiological wastes containing blood or other potentially infectious materials. 

Research Laboratory means a laboratory producing or using research-laboratory-scale amounts of HIV or HBV. Research laboratories may produce high concentrations of HIV or HBV but not in the volume found in production facilities. 

Sharps with engineered sharps injury protections means a nonneedle sharp or a needle device used for withdrawing body fluids, accessing a vein or artery, or administering medications or other fluids, with a built-in safety feature or mechanism that effectively reduces the risk of an exposure incident. 

Source Individual means any individual, living or dead, whose blood or other potentially infectious materials may be a source of occupational exposure to the employee. Examples include, but are not limited to, research participants; hospital and clinic patients; clients in institutions for the developmentally disabled; trauma victims; clients of drug and alcohol treatment facilities; residents of hospices and nursing homes; human remains; and individuals who donate or sell blood or blood components. 

Sterilize means the use of a physical or chemical procedure to destroy all microbial life including highly resistant bacterial endospores. 

Universal Precautions refers to a method of infection control in which all human blood and other potentially infectious materials are treated as if known to be infectious for HIV and HBV.  Id does not apply to feces, nasal secretions, sputum, sweat, tears, urine or vomitus unless they contain visible blood.     

Work Practice Controls means controls that reduce the likelihood of exposure by altering the manner in which a task is performed (e.g., prohibiting recapping of needles by a two-handed technique).

APPENDIX B

Spill Clean Up Procedures

  - Stop work.

  - If you are splashed by the material, dispose of PPE and wash hands.

  - Ensure that any other people in the vicinity are notified that a spill has occurred and that the room should be evacuated. Post a “Do Not Enter” notice on the door. Notify the PI or lab supervisor.

  - If you need assistance with the spill clean-up, call OEHS (1-6590).

  - Wait 60 minutes before re-entering the room to allow aerosols to settle.

  - Assemble Spill cleanup materials and don PPE, including lab coat, eye protection and face shield or mask, 2 pair of gloves, shoe covers.

  - Contain the spill by covering with paper towels (to avoid splashes or aerosols)

  - Saturate spill with disinfectant1 pouring from the outside in. Avoid splashing or generating aerosols.

      - For large spills (greater than 10ml) use undiluted bleach or disinfectant.

  - Wipe areas around the spill that may have splatter and any reusable equipment with disinfectant.

  - Let sit for the exposure time listed on the disinfectant container (20 minutes for bleach solutions).

  - Wipe up spill, disposing of towels in biohazard bag: if sharps may be present use tongs or a brush and pan and dispose in biohazard sharps container.

    - Work concentrically to clean up the absorbent material. Always work from the outer edge of the spill toward the center.

  - Spray spill area with disinfectant. Let sit for 10 minutes and wipe up as above, disposing of towels in biohazard bag

  - Remove PPE, discard disposable PPE as biohazardous waste and wash hands.

  - Remove the “Do Not Enter” sign and inform others that it is safe to re-enter the room.

1The disinfectant must be an EPA-approved disinfectant effective against HIV-1 and Hepatitis B virus (tuberculocidal recommended): lists can be found here.

APPENDIX C

# Hepatitis B Vaccine Declination

I understand that due to my occupational exposure to blood or other potentially infectious materials I may be at risk of acquiring hepatitis B virus (HBV) infection. I have been given the opportunity to be vaccinated with hepatitis B vaccine, at no charge to myself. However, I decline hepatitis B vaccination at this time. I understand that by declining this vaccine, I continue to be at risk of acquiring hepatitis B, a serious disease. If in the future I continue to have occupational exposure to blood or other potentially infectious materials and I want to be vaccinated with hepatitis B vaccine, I can receive the vaccination series at no charge to me.

### Date__________________________________

### Signature_____________________________

### Print Name_____________________________

APPENDIX D

# Sharps Injury Log

##### Laboratory Name:________________		Year 2_____

| Date | Case/Report # | Type of Device (e.g. syringe, suture needle) | Brand Name of Device | Work Area where injury occurred (e.g. Geriatrics, Lab) | Brief description of how incident occurred (i.e., procedure being done, action being performed (disposal, injection, etc.), body part injured) |
| --- | --- | --- | --- | --- | --- |

29 CRF 1910.1030, OSHA’s Blood borne Pathogens Standard, in paragraph (h)(5), requires an employer to establish and maintain a Sharps Injury Log for recording all percutaneous injuries in a facility occurring from contaminated sharps. The purpose of the Log is to aid in the evaluations of devices being used in healthcare and other facilities and to identify problem devices or procedures requiring additional attention or review. This log must be kept in addition to the injury and illness log required by 29 CFR 1904. The Sharps Injury Log should include all sharps injuries occurring in a calendar year. The log must be retained for five years following the end of the year to which it relates. The Log must be kept in a manner that preserves the confidentiality of the affected employee.